28BYJ-48 Dual Stepper Motor Control

ULN2003A Driver  •  Adafruit HUZZAH32 (ESP32-WROOM)  •  I2C Display

1. Overview

This guide covers control of two 28BYJ-48 stepper motors from an Adafruit HUZZAH32 (ESP32-WROOM32 Feather) using two separate ULN2003A driver chips. A small I2C display is also connected for status readout. All components share a common 3.3V logic level; the motor coils are powered from a 5V supply.

| Parameter | Value |
| --- | --- |
| Motor | 28BYJ-48, 5V unipolar stepper |
| Driver (×2) | ULN2003A, DIP-16 |
| Controller | Adafruit HUZZAH32 (ESP32-WROOM32) |
| Display interface | I2C (SDA = GPIO 21, SCL = GPIO 22) |
| Steps / rev (half-step) | 4096 |
| Gear ratio | 1:64 (approx) |
| Motor supply | 5V external — NOT the 3.3V board rail |

2. HUZZAH32 Pin Allocation

The HUZZAH32 uses an ESP32-WROOM32 module. Some board pins carry alternate labels (A0–A5) that do not match the underlying GPIO number — the table below shows both. Pins 21 and 22 are reserved for the I2C display and must not be used for other purposes.

| Board label | GPIO # | Assigned to | Notes |
| --- | --- | --- | --- |
| 21 | 21 | I2C SDA (display) | Default SDA in Arduino BSP |
| 22 | 22 | I2C SCL (display) | Default SCL in Arduino BSP |
| 14 | 14 | Motor 1 — IN1 | Clean output, no conflicts |
| 27 | 27 | Motor 1 — IN2 | Clean output, no conflicts |
| 32 | 32 | Motor 1 — IN3 | Clean output, no conflicts |
| 33 | 33 | Motor 1 — IN4 | Clean output, no conflicts |
| A1 | 25 | Motor 2 — IN1 | Board label A1, actual GPIO 25 |
| A0 | 26 | Motor 2 — IN2 | Board label A0, actual GPIO 26 |
| RX | 16 | Motor 2 — IN3 | Board label RX — safe if Serial1 unused |
| TX | 17 | Motor 2 — IN4 | Board label TX — safe if Serial1 unused |
| 34, 35, 36, 39 | 34,35,36,39 | Do not use for output | Input-only pins — no output driver |
| 6–11 | 6–11 | Do not use | Internal SPI flash — never expose |

Note: The A0/A1 labels on the HUZZAH32 silk refer to GPIO 26 and 25 respectively — not GPIO 0 and 1. Always use the GPIO number in code, not the A-prefix label.

Note: RX (GPIO 16) and TX (GPIO 17) are perfectly usable as general outputs as long as you do not need the hardware Serial1 port.

3. Motor Connector —Wire Colours

The 28BYJ-48 uses a 5-pin JST PH (2 mm pitch) connector. Pin numbering below is with the latch/tab facing you, left to right. Both motors use identical connectors and wire colours.

| Pin | Colour | Coil | ULN OUT pin | Connect to |
| --- | --- | --- | --- | --- |
| 1 | Blue | B2 (IN4) | Pin 13 | Motor 1: GPIO 33 / Motor 2: GPIO 17 (TX) |
| 2 | Pink | B1 (IN3) | Pin 14 | Motor 1: GPIO 32 / Motor 2: GPIO 16 (RX) |
| 3 | Yellow | A2 (IN2) | Pin 15 | Motor 1: GPIO 27 / Motor 2: GPIO 26 (A0) |
| 4 | Orange | A1 (IN1) | Pin 16 | Motor 1: GPIO 14 / Motor 2: GPIO 25 (A1) |
| 5 | Red | +5V common | — | +5V supply + ULN2003A pin 9 (COM) |

Note: The red wire must connect to both +5V and the ULN2003A COM pin (pin 9). This activates the internal flyback diodes which suppress voltage spikes when coils switch off.

4. ULN2003A Chip Pinout (DIP-16)

Wire both ULN2003A chips identically — only the GPIO source pins differ between Motor 1 and Motor 2.

| Pin | Signal | Pin | Signal | Notes |
| --- | --- | --- | --- | --- |
| 1 | IN1 ← GPIO (see §2) | 16 | OUT1 → Orange wire | Coil A1 |
| 2 | IN2 ← GPIO (see §2) | 15 | OUT2 → Yellow wire | Coil A2 |
| 3 | IN3 ← GPIO (see §2) | 14 | OUT3 → Pink wire | Coil B1 |
| 4 | IN4 ← GPIO (see §2) | 13 | OUT4 → Blue wire | Coil B2 |
| 5 | IN5 (unused) | 12 | OUT5 (unused) |  |
| 6 | IN6 (unused) | 11 | OUT6 (unused) |  |
| 7 | IN7 (unused) | 10 | OUT7 (unused) |  |
| 8 | GND | 9 | COM → +5V | Must connect to motor supply |

5. Full Wiring Summary

Motor 1

| HUZZAH32 label | GPIO # | ULN2003A #1 | Motor 1 wire |
| --- | --- | --- | --- |
| 14 | 14 | Pin 1 (IN1) → Pin 16 (OUT1) | Orange |
| 27 | 27 | Pin 2 (IN2) → Pin 15 (OUT2) | Yellow |
| 32 | 32 | Pin 3 (IN3) → Pin 14 (OUT3) | Pink |
| 33 | 33 | Pin 4 (IN4) → Pin 13 (OUT4) | Blue |
| +5V | — | Pin 9 (COM) | Red (common) |
| GND | — | Pin 8 (GND) | — |

Motor 2

| HUZZAH32 label | GPIO # | ULN2003A #2 | Motor 2 wire |
| --- | --- | --- | --- |
| A1 | 25 | Pin 1 (IN1) → Pin 16 (OUT1) | Orange |
| A0 | 26 | Pin 2 (IN2) → Pin 15 (OUT2) | Yellow |
| RX | 16 | Pin 3 (IN3) → Pin 14 (OUT3) | Pink |
| TX | 17 | Pin 4 (IN4) → Pin 13 (OUT4) | Blue |
| +5V | — | Pin 9 (COM) | Red (common) |
| GND | — | Pin 8 (GND) | — |

I2C Display

| HUZZAH32 label | GPIO # | Connect to |
| --- | --- | --- |
| 21 (SDA) | 21 | Display SDA |
| 22 (SCL) | 22 | Display SCL |
| 3V | — | Display VCC (if 3.3V display) |
| GND | — | Display GND |

Note: Most small I2C OLED displays (SSD1306, SH1106) run on 3.3V and are compatible directly. Add 4.7 kΩ pull-up resistors on SDA and SCL to 3.3V if your display module does not already include them.

6. Half-Step Sequence

The same 8-phase sequence applies to both motors. Each row shows the coil states driven by IN1–IN4.

| Step | IN1 (Orange) | IN2 (Yellow) | IN3 (Pink) | IN4 (Blue) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 |
| 2 | 0 | 1 | 0 | 0 |
| 3 | 0 | 1 | 1 | 0 |
| 4 | 0 | 0 | 1 | 0 |
| 5 | 0 | 0 | 1 | 1 |
| 6 | 0 | 0 | 0 | 1 |
| 7 | 1 | 0 | 0 | 1 |

7. ESP32 Arduino / PlatformIO Code

The Stepper class below is self-contained — no external libraries needed. Instantiate once per motor with its four GPIO pins. Both motors can be stepped independently. GPIO numbers match the HUZZAH32 pin allocation table in section 2.

#include <Arduino.h>  class Stepper28BYJ { public:     Stepper28BYJ(int p1, int p2, int p3, int p4)         : pins{p1, p2, p3, p4}, step(0) {         for (int p : pins) pinMode(p, OUTPUT);     }      // direction: +1 = CW,  -1 = CCW     // delayMs  : step interval (min ~2 ms)     void rotate(int steps, int dir, int delayMs = 3) {         for (int i = 0; i < steps; i++) {             step = (step + dir + 8) % 8;             apply();             delay(delayMs);         }     }      void off() {         for (int p : pins) digitalWrite(p, LOW);     }  private:     int pins[4];     int step;     static constexpr bool seq[8][4] = {         {1,0,0,0},{1,1,0,0},{0,1,0,0},{0,1,1,0},         {0,0,1,0},{0,0,1,1},{0,0,0,1},{1,0,0,1}     };     void apply() {         for (int i = 0; i < 4; i++)             digitalWrite(pins[i], seq[step][i]);     } };  // ---- Motor 1: board labels 14, 27, 32, 33 ---- Stepper28BYJ motor1(14, 27, 32, 33);  // ---- Motor 2: board labels A1, A0, RX, TX ---- //              actual GPIO:  25, 26, 16, 17 Stepper28BYJ motor2(25, 26, 16, 17);  void setup() { /* I2C display init goes here */ }  void loop() {     motor1.rotate(4096, +1);  // Motor 1: 1 rev CW     motor1.off();      motor2.rotate(2048, -1);  // Motor 2: half rev CCW     motor2.off();      delay(1000); }

8. Practical Tips

- Always call off() when a motor is stopped. The coils stay energised otherwise, the motor runs warm and draws current for no reason.

- Power the motor coils from an external 5V supply. The HUZZAH32 3.3V rail cannot supply the ~200 mA per coil that the motors need.

- Share GND between the ESP32, both ULN2003A chips, the motors, and the 5V supply.

- 4096 half-steps = 360 degrees. 2048 = 180 degrees. 341 = 30 degrees (approx).

- The minimum reliable step delay is ~2 ms. At 3 ms the motion is smoother and quieter with little speed penalty.

- The two motors in this code run sequentially. For simultaneous motion, use a millis()-based non-blocking stepper loop or FreeRTOS tasks on the two ESP32 cores.

- For the I2C display: if your module lacks onboard pull-ups, add 4.7 kΩ resistors from SDA (GPIO 21) and SCL (GPIO 22) to the 3.3V rail. Many small OLED breakouts already include these.

- GPIO 16 and 17 (board labels RX/TX) are used here for Motor 2. If you later need the hardware Serial1 port, reassign those motor pins to other free GPIOs (e.g. GPIO 13 and one of the SPI pins if SPI is also unused).